IMU-Based Step and Orientation Detection: A Review

Tania Pan
Dept. of Electrical, Computer and Software Engineering
University of Auckland
Auckland, New Zealand

Abstract—This review compares six approaches to detecting steps and heading from inertial sensor data for pedestrian dead reckoning, and identifies those suited to a device with only an accelerometer and a magnetometer.

Keywords—pedestrian dead reckoning, step detection, heading estimation, accelerometer, magnetometer

# Introduction

Pedestrian dead reckoning (PDR) adds a displacement to the previous position each time a step is detected [1], [2]. It avoids double integration of acceleration, whose error grows quickly unless a foot-mounted sensor allows zero-velocity updates [1], [3]. Step and heading errors still accumulate, and heading is repeatedly reported as the dominant one [3]–[5]. This review compares six works with a view to a wearable device that has only an accelerometer and a magnetometer.

# Step Detection

The detectors differ first in the signal examined. Three works rotate acceleration into a world frame and use the vertical component, making detection independent of device posture [1], [4], [5]. Jiménez et al.[3] and Brajdic and Harle [6] use the acceleration magnitude instead, the latter because the vertical axis is hard to isolate on a loosely carried phone. Tian et al. classify how the phone is carried (held, swinging or in a pocket) and select one axis per mode, since no single threshold on the magnitude fits all modes [2].

Two works threshold the local variance of acceleration: on a foot-mounted sensor this missed 1 of 955 steps [3], and in a trouser pocket it gave under 1% count error, though only for the steps of one leg [4]. The others detect peaks in low-pass filtered acceleration. Jin et al. require a maximum and minimum to differ by a calibrated threshold and be at least 150 ms apart [1]; Kang and Han test peak height, peak-to-valley difference and slope [5]; Tian et al. use a fixed threshold, a 0.2 s minimum step interval and a false-peak rejection window, detecting over 98% of steps [2]. Comparing ten algorithms on 130 walks by 27 people with six phone placements, Brajdic and Harle found all had median errors under 3%, and windowed peak detection matched hidden Markov and wavelet methods at 1.3% while being much simpler [6]. Steps are hardest to detect at the start and end of a walk [3], [6].

# Orientation Detection

Heading is obtained in three ways. A magnetometer gives absolute heading once the magnetic vector is rotated into the horizontal plane, since uncompensated tilt causes large errors [5], but indoors the field is distorted by metal furniture, computers and machinery [1], [3], [5]. A gyroscope is immune to this but drifts after a few turns [5]; Tian et al. average its yaw over each step and train an offset when the phone does not point along the path [2]. Kang and Han weight the previous estimate, magnetometer and gyroscope by how well the two sensors agree, reaching a 2.28 degree mean heading error [5]. Third, when the device orientation on the body is unknown, walking direction is inferred from horizontal acceleration: principal component analysis was the best of five methods in [4], with a 5.7 degree median error, but Jin et al. found it unreliable below 25 Hz and integrated horizontal acceleration over 1.4 s instead [1].

# Comparison and Implications

Table I summarises the approaches. For a device worn in a fixed orientation, the literature supports peak detection on low-pass filtered acceleration with an amplitude threshold and a minimum step interval, calibrated for the wearer [1], [2], [6]. Without a gyroscope, heading must come from a tilt-compensated magnetometer [5], so indoor magnetic disturbance is the main expected limitation.

Comparison of Reviewed Approaches

| Ref. | Sensors, placement | Step detection | Heading |
| --- | --- | --- | --- |
| [1] | Acc., compass; two phones in pockets | Vertical acc.; max-min pair, 150 ms gap | Integrated horizontal acc.; device fusion |
| [2] | Acc., gyro; hand, swinging, pocket | Axis chosen by mode; peak, 0.2 s gap | Gyro yaw averaged per step, trained offset |
| [3] | Acc., gyro, mag.; foot | Variance of acc. magnitude, two thresholds | Fused IMU orientation at foot stance |
| [4] | Acc., gyro, mag.; trouser pocket | Variance of vertical acc., threshold | PCA of horizontal acc. |
| [5] | Acc., gyro, mag.; phone in hand | Vertical acc.; peak, peak-to-valley, slope | Tilt-compensated mag. weighted with gyro |
| [6] | Acc.; six phone placements | Acc. magnitude; windowed peak detection | Not covered |

##### References

Y. Jin, H.-S. Toh, W.-S. Soh, and W.-C. Wong, “A robust dead-reckoning pedestrian tracking system with low cost sensors,” in Proc. IEEE Int. Conf. Pervasive Computing and Communications (PerCom), Seattle, WA, USA, Mar. 2011, pp. 222–230.

Q. Tian, Z. Salcic, K. I.-K. Wang, and Y. Pan, “A multi-mode dead reckoning system for pedestrian tracking using smartphones,” IEEE Sensors J., vol. 16, no. 7, pp. 2079–2093, Apr. 2016.

A. R. Jiménez, F. Seco, C. Prieto, and J. Guevara, “A comparison of pedestrian dead-reckoning algorithms using a low-cost MEMS IMU,” in Proc. IEEE Int. Symp. Intelligent Signal Processing (WISP), Budapest, Hungary, Aug. 2009, pp. 37–42.

U. Steinhoff and B. Schiele, “Dead reckoning from the pocket – an experimental study,” in Proc. IEEE Int. Conf. Pervasive Computing and Communications (PerCom), Mannheim, Germany, Mar. 2010, pp. 162–170.

W. Kang and Y. Han, “SmartPDR: Smartphone-based pedestrian dead reckoning for indoor localization,” IEEE Sensors J., vol. 15, no. 5, pp. 2906–2916, May 2015.

A. Brajdic and R. Harle, “Walk detection and step counting on unconstrained smartphones,” in Proc. ACM Int. Joint Conf. Pervasive and Ubiquitous Computing (UbiComp), Zurich, Switzerland, Sep. 2013, pp. 225–234.